### SpotMicro Minimalist Project - Egypt (Detailed)

### Executive Summary

This proposal outlines a simplified SpotMicro quadruped robot focused on three core features: obstacle avoidance, movement primitives, and a remote controller interface. The project will be built in Cairo, Egypt, using locally available components, with a total hardware budget of 6,814 EGP.

### Core Features

- Autonomous steering: Utilizes an ultrasonic sensor as well as stereo cameras to be able to steer autonomously while dodging obstacles

- Controller Interface: Allows for remote operation via a physical RC controller or MQTT commands.

### Hardware Components

#### Mechanical Platform

- Model: SpotMicro V4 (Community Model from CrealityCloud: https://www.crealitycloud.com/model-detail/spotmicro-robotic-dog-v4)

- Materials: PLA 3D-printed parts, to be fabricated by a local print shop.

- Printer: Creality Ender 3

- Infill: 15%

- Colors: White, Black, Gray

- Actuation: Twelve high-torque MG996R servo motors (three per leg) provide comprehensive control over the robot's locomotion.

#### Electronics & Control

| Component | Model | Price (EGP) | Function & Description | Source (Link) |
| --- | --- | --- | --- | --- |
| Microcontroller | Arduino Nano RP2040 Connect | 1,750 | Main control board with onboard Wi-Fi for MQTT communication and an IMU for balance. | RAM Electronics: Link |
| Servo Driver | PCA9685 16-Channel PWM | 210 | I²C-controlled driver for precise, synchronized control of all twelve servos. | MakerSelectronics: Link |
| Servos | MG996R Metal-Gear (×12) | 2,520 | High-torque servos providing powerful and precise leg actuation for dynamic movement. | MakerSelectronics: Link |
| Ultrasonic Sensor | HC-SR04 | 60 | Provides distance measurement for forward obstacle detection and avoidance. | MakerSelectronics: Link |
| Stereo Cameras | OV9655 (×2) | 700 | A pair of cameras used for depth perception | RAM Electronics: Link |
| Battery | 12V Li-ion 7000mAh | 1,200 | Powers the entire system, including the microcontroller and all twelve servos. | (Generic Estimate) |
| Display | SSD1306 OLED 0.96" | 170 | Small screen for providing real-time debug information and status feedback. | MakerSelectronics: Link |
| DC to DC step down BUCK converter | LM2596S | 50 | Power the logic microcontroller 12V down to 5V | https://www.ram-e-shop.com/shop/dc-dc-100-dc-dc-step-down-voltage-converter-adjustable-3a-lm2596s-4-38vdc-to-1-5-35vdc-sku-dc100-7257 |
| Step Down Buck Converter Power Supply XL4016 PWM Adjustable 4-40V To 1.25-36V Step-Down Board Module XH-M401 | XL4016 PWM | 150 | 12V down to 6V for your 12 servos. | https://www.ram-e-shop.com/shop/dc-dc-105-dc-dc-step-down-voltage-converter-adjustable-8a-xl4016-4v-40vdc-to-1-25v-36vdc-sku-dc105-7670 |
| Rocker Switch (10A) | Switch | 4 | To switch on and off the robot | https://makerselectronics.com/product/rocker-switch-spdt-2-position-on-off/ |
| Small 5V DC Motor (130-size) | DC motor | 50 | Used to actuate a simple "wagging tail" for feedback. |  |
| Total |  | 6,814 EGP |  |  |

## 3. Subsystems & Operational Logic

This project is broken into four main subsystems, designed to meet the course requirements (3 subsystems, 2 parallel).

- Subsystem 1: Core Balance & Locomotion (Sequential)

- Subsystem 2: Autonomous Navigation (Parallel)

- Subsystem 3: Remote MQTT Control (Parallel)

Subsystems 2, and 3 are designed to run in parallel. The robot can be autonomously navigating (Sub 2) while simultaneously listening for MQTT command (Sub 3).

### Subsystem 1: Core Balance & Locomotion (Sequential)

- Story Overview: The robot's core function is to maintain balance while standing or walking, using an IMU to correct its posture.

- Inputs: LSM6DSOXTR 6-axis IMU (Analog-style data stream).

- Outputs: 12x MG996R Servo Motors, SSD1306 OLED Display.

- Overall Logic:

  - Initiation: Robot is powered on via the Rocker Switch. The OLED Display shows "BOOTING...".

  - Calibration: The system initializes the LSM6DSOXTR IMU. The OLED displays "CALIBRATING IMU... DO NOT MOVE".

  - Ready State: After 3 seconds, calibration is complete. The robot moves to a default "STAND" pose. The OLED displays "STATUS: IDLE".

  - Sequential Loop (Balancing): The system continuously runs this loop:

    - Read the current pitch and roll from the IMU.

    - IF |pitch| > 10 OR |roll| > 10 degrees: The system calculates the micro-adjustments needed for the 12 leg servos to counteract the tilt and restore balance.

    - The PCA9685 sends the new commands to the servos.

  - Command Execution: When a command is received from another subsystem (e.g., "WALK_FWD"), this subsystem executes the gait cycle, while the "Sequential Loop (Balancing)" continues to run to ensure stability during movement.

### Subsystem 2: Autonomous Navigation (Parallel)

- Story Overview: In "Patrol Mode," the robot uses its sensors to navigate a space, avoiding simple (walls).

- Inputs: HC-SR04 Ultrasonic Sensor, 2x OV9655 Stereo Cameras, LSM6DSOXTR 6-axis IMU

- Outputs: SSD1306 OLED Display, (Sends commands to Subsystem 1).

- CV: Stereo vision depth mapping for complex obstacle/drop-off detection.

- Overall Logic:

  - Activation: The robot receives a "PATROL" command (from Subsystem 3 or 4). The OLED displays "STATUS: PATROLLING".

  - Default State: The system sends a "WALK_FWD" command to Subsystem 1.

  - Sensing Loop (Parallel): The system continuously runs two sensing tasks at the same time:

    - Task A (Ultrasonic): The HC-SR04 continuously pings for immediate frontal obstacles.

    - Task B (Stereo Vision): The 2x OV9655 Cameras continuously process frames to build a basic depth map of the environment,

    - Task C (IMU): state estimation to check where the robot is

  - Conditional Logic (Obstacle Detected):

    - IF Stereo Vision detects a drop-off: This is a high-priority interrupt. The system sends "EMERGENCY_STOP" then "TURN_180" to Subsystem 1. The OLED displays "DANGER: DROP-OFF!".

    - IF HC-SR04 distance < 20cm (Simple Wall): The system sends "STOP" then "TURN_LEFT_90" to Subsystem 1. The OLED displays "STATUS: AVOIDING".

  - Resumption: After a "TURN_LEFT_90" command, the IMU (Task C) verifies the robot has turned ~90 degrees before returning to "WALK_FWD".

### Subsystem 3: Remote MQTT Control (Parallel)

- Story Overview: The robot can be fully controlled and monitored remotely over Wi-Fi via an MQTT dashboard, allowing for high-level commands and status checks.

- Inputs: Arduino Nano RP2040 Connect (Wi-Fi Module).

- Outputs: (Sends commands to all other Subsystems).

- Web Interface: Fulfills the "Web Interface" requirement via MQTT protocol.

- Overall Logic:

  - Initiation: On boot (after Subsystem 1 is "IDLE"), the RP2040 connects to the designated Wi-Fi network and MQTT broker. The OLED displays "MQTT CONNECTED".

  - Listening Loop (Parallel): The system subscribes to the spotmicro/command topic and listens in the background.

  - Command Execution:

    - IF Message == "PATROL": Activates Subsystem 2.

    - IF Message == "STOP": Sends "STOP" to Subsystem 1.

    - IF Message == "TAIL_WAG": Triggers the DC motor.

    - IF Message == "STATUS_REQUEST": The robot gathers data (IMU status, Obstacle status) and publishes it to the spotmicro/status topic.

    - IF Message == “TURN_LEFT_X”: The robot will turn left with the value X degrees

    - IF Message == “TURN_RIGHT_X” The robot will turn right with the value X degrees

    - IF Message == “WALK_FWD” The robot will move forward

    - IF Message == “WALK_BWD” The robot will move backward